Chinn			OWP 2023			2448 76th Ave SE

Legacy Mercer Island Apartments	 		Mercer Island, 98040

Description of Materials  with CO# 01

# Resident unit trim

# 230	white Toto CST416M dual flush elongated bowl toilet w/Toto SS114 seat

# 11	white Toto CST744EL, 17” high ADA elongated toilet w/Toto SS114 seat

#### 241	white Toto LT502 china lavatory sink w/Grohe 34.270.001 chrome faucet

#### 138	white Superior 101C tub/shower stall w/backing w/Moen 62365/TL183EP chrome valve

#### 11	white Superior 100H tub/shower stall w/grab bars and seat w/Moen 62365 /TL183EP chrome

#### 11	chrome Moen 52710EP-1.5 hand held shower at type A unit bathtubs

#### 28	white Superior 48C-N shower stall (not adaptable) w/Moen 62365 /TL182EP chrome

#### 64	white Superior 48BF-FWA shower stall (adaptable) w/Moen 62365 /TL182EP chrome

#### 198	Moen G20192, 21”x16”x7.5” undercounter stainless steel sink w/Moen 7594 chrome

#### 11	Moen G18195, 21”x16”x5.5” ADA undercounter stainless steel sink w/Moen 7594 chrome

#### 209	roughin and connect for unit dishwasher (appliance to be FBO)

# 209	icemaker roughin with Sioux Chief recessed plastic fire rated box

# 209	washer roughin with Sioux Chief recessed plastic fire rated box

###### Common areas

# 3	white Toto CST744EL, 17” high ADA elongated toilet w/Toto SS114 seat

#### 3	white Toto LT502 china lavatory sink w/Grohe 34.270.001 chrome faucet

#### 2	Moen G18195, 21”x16”x5.5” ADA undercounter stainless steel sink w/Moen 7594 chrome

###### Building systems

# 10	HB-1 Woodford 25 keyless hose bib in garage or mechanical rooms

# 10	HB-2 Woodford 65B frostproof keyless hose bib with recessed box at exterior locations

# 1	HB-3 Woodford 25 keyless hose bib with hot and cold water supply at level P trash room

# 3	Mustee 63M mop sink w/Mustee wall hung faucet at levels P, 1 and 2 janitor rooms

4	AO Smith BTH500, 130-gallon gas fired water heaters with PVC vent piping

1    	Armstrong hot water expansion tank

Powers or equal hot water mixing valves

2   	Armstrong E-series hot water circulating pumps w/Tekmar pump control

1	HTP Armstrong/Gemini duplex variable speed domestic water pressure booster package

Watts backflow valve at domestic water supply

1     Watts backflow valve at retail area water supply

1	irrigation water supply to POC at level L3 deck(excludes irrigation valves or pipe under pavers)

1	trash chute water supply with Watts backflow valve

1	Quality Concrete 36”x36” concrete sand/oil basin for parking garage floor drainage

6	Quality Concrete CB40, 12”x18” concrete garage floor drain at level P slab on grade

2	Quality Concrete type 241 concrete basin for silt collection from foundation drains

1	Quality Concrete 48” round pump basin for contaminated soil system

1	Zoeller 161 Duplex sump pump system for contaminated soil system (excl flow restrictor)

2	NDS 090N trench drain with DS231 cast iron grating at garage and loading bay entry

9	ProSet or Watts cast iron garage floor drains at level L1

15	ProSet or Watts cast iron floor drains and hub drains at mechanical and trash rooms

1	PVC standpipe drain in fire sprinkler room

5	Watts cast iron deck drains at level 2 post tension deck

40	Watts cast iron deck drains at level 3 post tension deck

17	Watts cast iron roof drains with overflow at rooftop

1	PVC interior foundation and sub-slab drain pipe inside foundation wall as shown on page C301

1	single common gas supply for (3) water heaters, (4) rooftop HVAC (1) rooftop BBQ, (1) fireplace

6	4” waste, 3” vent and 1.5” cold water stub to each TI space.

1	8” vent line from P-025 to roof.  Excludes fans, ports, valves, etc.

Chinn			OWP 2023			2448 76th Ave SE

Legacy Mercer Island Apartments	 		Mercer Island, 98040

INCLUDED IN BASIC PLUMBING BID

Foundation wall and sub-slab perforated drain piping to be solid core PVC plastic pipe (excludes

drain rock, drainage mat, filter fabric and Miradrain fittings at the foundation wall).

Waste and storm drain piping to be solid core PVC plastic pipe under slab on grade.

All vent piping throughout building to be solid core PVC plastic pipe and fittings.

Waste and drain stacks in parking garage and mechanical areas to be solid core PVC plastic.

Waste piping within retail and amenity space ceilings and walls to be solid core PVC plastic.

Waste and storm drain stacks in unit walls to be solid core PVC pipe.

Water piping to be CPVC piping for 2.5” and larger pipe with 2” and smaller PEX pipe.

Insulation to code on hot water recirculation supply and return mains

Gas piping to be threaded black iron pipe and fittings.

PVC vent piping for water heater venting.

Waste, drainage & water mains to tie onto utility stubs left by utility contractor at foundation wall.

Hot/Cold water unit water shut off valves(submeters or access panels to be furnished by others).

ProSet sleeve system or equal to be used at all post tension decks only.

City of Mercer Island plumbing permit and plan review fees.

EXCLUDED FROM BASIC PLUMBING BID

Excavation, backfill, fill materials, pea gravel, sand or drain rock inside or outside of building.

Placement of concrete basins in slab on grade.  Export of trench spoils.

Drain mat or adaptors at foundation walls.

Storm detention systems.

Grease interceptors.  Grease waste piping.

Storm, sanitary, foundation drain or planter drain piping outside of foundation walls.

Electrical supply or wiring for water heaters, recirc pumps, sump pumps.

Emergency electrical shutoffs by others.

Fire sprinkler system.

Fire stopping or caulking in wood frame construction.

Exterior canopy drains, pickups or downspouts

Housekeeping base for hot water heaters.

Flashing at building siding for hosebibs, water heater vents.

acoustic insulation on waste, vent, drain or water piping

Thermal insulation on water piping to meet code.

Wash machine drain pans.  Kitchen sink garbage disposals.

Access panels.

City water meters/services or unit water meters.  Gas meters.

Landscape or planter irrigation system, controls or valves.

Exterior side sewers, storm sewers or water service.

Waste disposal fees.

Retail area gas piping.

Hoisting or crane picks of fiberglass stalls/roughin materials during roughin

Chinn			OWP 2023			2448 76th Ave SE

Legacy Mercer Island Apartments	 		Mercer Island, 98040

Labor codes:

1	Supervision

2	Groundwork labor (includes waste, vent, water or foundation/storm drainage)

3	Waste, vent, storm drain or water labor in the residential units

4	Finish or trim labor

5	Gas piping labor

6	Waste, storm drain, condensate drain or water labor above slab on grade within the parking garage or retail space

7	Proset or can out labor

12	Layout and takeoff

13    	water main and insulation labor

14    	Mechanical room labor

Material codes:

30	Groundwork material (includes waste, vent, water or foundation/storm drainage)

31	Waste, vent or water material in the residential units

32	Finish or trim materials

33	Gas piping material

34		Waste, storm drain, condensate drain or water material above slab on grade   within  the parking garage or retail spaces

35	Proset, can out materials

40   	water main and insulation material

41   	Mechanical room materials

Other codes:

60	  subcontractor

61	  engineering

62	  rental equipment

63	  permit and review fees

67	  Parking and other expenses