May 6, 2015

Chinn Construction, LLC

14715 NE 95th Street, Suite 101

Redmond, WA 98052

Attn: 	Lauren Bjorndahl

Re:	Hadley Apartments – Plumbing Fixture Submittal Narrative

The intent of this memo is to address items requiring attention in the plumbing fixture submittals.

- TS-1 Tub/Shower: It is our understanding that the type B unit bathtub/shower stalls are supposed to be ‘adaptable’, which means that it needs to be capable of receiving grab bars.  The Superior 101C does not have a flat back wall.  If grab bars are eventually installed, there will not be a consistent 1-1/2” dimension between the backside of the grab bar and the wall.  Please review for adaptable compliance.

- Reference details listed below.  Details visually depict a wall hung lav in the leasing and fitness restrooms, while the amenity restroom shows a drop-in lav.  Our contract calls for the Toto drop-in lav at all locations, but we can provide Kohler K2007 if wall hung lavs are desired.  Please select.

  - Leasing Restroom: 2/A0221B, 1/ID-02, 5/ID-15

  - Fitness Restroom: 2/A0223A, 1/ID-08, 5/ID-18

  - Skylounge Restroom: 11/A0600, 2/ID-08, 2/ID-20

- Reference details listed above.  The Architectual and ID plan view details visually depict a tank-type, floor-mount toilet.  The ID elevation view details visually depict a flushometer, wall-mount toilet.  Our contract is for the Toto CST744EL tank-type, floor-mount toilet.  Please confirm this is acceptable.  If flushometer, wall-mount toilets are desired we can submit a suggestion with pricing.  Please note, wall mount toilet will require minimum 8” wall for carrier install.

- Architectural/Interior drawings show double compartment kitchen sinks in some units.  Contract  specification is for single compartment sinks.  If double compartment sinks are desired we can submit a suggestion with pricing.

Please let me know if you have any questions or require further information.  Thank you.

Sincerely,

Rich Donelson

Project Manager / Estimator

LEED® Green Associate

richd@owpllc.net

425-968-8314 ext-118

425-591-4296 cell