OWP 2025			THE CORA		19 West Harrison Street

Exxel Pacific			    	 			Seattle, 98119

# Resident unit trim

# 84	white Gerber 21-912, elongated bowl toilet with Bemis 1500 elongated seat

# 8	white Toto CST244, 17” high, ADA elongated toilet or equal w/Bemis 1500 elongated seat

#### 92	white Toto LT221 undercounter lav w/Grohe 34-270.001 chrome faucet

#### 60	white Aquatic 2603SMTE, fiberglass tub/shower stall w/Delta R10000MF/T14459 chrome

#### 6	white Aquatic 2603SMTE, type A w/grab bars w/folding seat w/Delta R10000MF/T14459

#### 24	white Aquatic 1483EN shower stall w/Delta R10000MF/T14259 chrome

#### 4	Delta RPW336HDF chrome hand held shower at type A units (no handshowers at Live/work)

#### 67	Lenova LS18304 undercounter stainless steel kitchen sink w/Grohe 32-665.001 chrome faucet

#### 8	Elkay ELUHAD2115-5 undercounter stainless steel sink w/ Grohe 32-665.001(at type A units)

# 75	roughin and connect for unit dishwasher (appliance to be FBO)

# 75	washer roughin w/ recessed box, valves & condensate adapter(excludes install of appliance)

# 4	icemaker boxes at ADA units in wood framed building

###### Amenity/retail restrooms, lobby sink,  maintenance room, Activity room

# 2	white Toto CST244,  ADA elongated toilet w/Bemis 1955 open front seat

# 1	white Toto LT307 wall-hung lav w/ carrier w/Grohe 34.271.001 chrome w/grid strainer at fitness

# 1	white Toto LT307 wall-hung lav w/ carrier w/Grohe 34.271.001 chrome w/grid strainer at retail

# 1	Elkay ELUHAD2115-5 undercounter stainless steel sink w/ Delta 9959-CZ-DST bronze, activity rm

# 1	Icemaker and dishwasher in activity room

# 1	EEmax on-demand water heater to serve the retail restroom lavatory

# 2	Watts floor drains in amenity and retail restrooms with trap seal

# 1	Lenova PC-SS-ORI-S15 undercounter stainless steel bar sink w/Grohe 32-665.001 chrome

#### 1	Mustee 63 mop sink with Mustee wall hung chrome faucet at janitor closet (room 215)

# 2	Watts floor drain in bike storage rooms (rooms 111 and 207) with trap seal

# Building systems

1	Backwater valve and waste piping to serve fixtures below manhole rim height at 101.45

1	waste, vent and domestic cold water stub to retail space

2	Woodford 24 keyless hosebib at P105 and P106 mechanical rooms

# 4	Woodford 65 frostproof keyless hose bib for exterior wall locations

# 1	Woodford 24 keyless hosebib w/hot/cold water at residential trash (room 211)

# 1	Woodford hose bib or yard hydrant near dog run area at roof deck

# 1	T&S Brass PG-8WREV wall hung hot/cold faucet w/flex hose and hand spray at dog wash (level P1)

2	PVI Conquest 40L130A, gas-fired condensing water heaters with PVC vent piping

1    	Armstrong expansion tank

2   	Armstrong hot water circulating pumps with single Tekmar controller

1     Watts backflow valve at domestic water supply

1     duplex domestic water booster pump

# 1	Watts FD15-6R-B floor drain w/6” strainer and sediment bucket at dog wash area (level P1)

# 2	Proset or Watts floor drain in water entry room (P105) and water heater room (P106)

# 2	PVC hub drains in water entry room (P105) for fire sprinkler blowoff and domestic RPBA drain

# 2	PVC hub drains in water heater room (P106) for water heater condensates

# 1	PVC hub drain for elevator AC condensate at level P1

# 3	PVC hub drains as needed for AC condensates for lobby, fitness and commercial space

1	4” schedule 40, DWV PVC foundation wall drain around interior of basement foundation walls

1	Zoeller duplex sump pump with basin for foundation drains

1	Zoeller duplex explosion proof sump pumps with basin for garage floor drains

1	PVC control assembly shown on detail 2, page C0310. (excludes vault, manhole, ladder)

1	Quality Concrete 36”x36” concrete sand/oil basin for parking garage floor drainage

3	Quality Concrete CB40, 12”x12” concrete garage floor drain at slab on grade

2     ProSet P65X drain with cast iron grate for residential and commercial trash area

OWP 2025			THE CORA		19 West Harrison Street

Exxel Pacific			    	 			Seattle, 98119

2	ProSet P65X drain with cast iron grate for level 2 garage parking area

2     NDS DS-090N trench drains w/DS231 grates at level 1 and level 2 garage ramps

4     NDS DS-090N trench drains w/DS226 grates at west side of building per Civil drwgs

1	storm drain pickup from west side exterior storm drainage shown on civil plan page C0300

2	Watts cast iron area drain w/overflow at level 1 (at NW corner outside of retail space)

2	Watts cast iron area drains w/overflow at level 2 (entry area of live/work units C and D)

7	Watts cast iron deck drain w/overflow at L3 deck (4 at courtyard, 1 at NE corner, 1 west of stairwell #2)

10	Watts cast iron roof drains with overflow at roof

1	single common irrigation water supply to point of connection at street level, 3rd floor and roof decks.

1	single common gas supply for (2)HWT’s, (1) lobby fireplace, (1)BBQ, (1)fire-pit, (1) HVAC at roof deck

NOTE: Only HOT water will be metered.  OWP to install ¾” spacer tube in vertical or horizontal orientation on HOT.  Cold is to be solid pex with single shutoff.  Meter install by others.  This was changed via CO #4.

Note:  Proset sleeves to be installed in post tension decks after all wall locations have been snapped or marked on deck forms by others and prior to placement of steel and cables.

INCLUDED IN BASIC PLUMBING BID

PVC pipe is to be solid core to meet BOD plumbing specification.

Foundation drain piping around perimeter of exterior foundation wall only. Piping to be 4” PVC plastic pipe

(excludes drain rock, filter fabric, drainage mat and miradrain fittings at the foundation wall).

All vent piping throughout building to be PVC plastic pipe.

Waste pipe in retail and amenity areas to be cast iron.

Waste pipe to be PVC plastic pipe in parking garage, bike parking, underground and mechanical spaces.

Waste stacks in unit walls and ceilings to be cast iron pipe (fixture trap arms and toilet flanges to be PVC).

Storm drain piping to be PVC plastic pipe in parking garage and under slab on grade.

Roof drain stacks to be cast iron within units, retail and amenity space.  Overflow drain pipe to be plastic pipe.

Water piping to be plumbed with PEX pipe up to 3” size and 2.5” and larger CPVC pipe to code.

Thermal insulation to code on hot water recirculation supply and return mains only.

Gas piping to be threaded black iron pipe and fittings (no welded pipe).

Waste, drainage and water mains to tie onto utility stubs left by utility contractor at foundation wall.

Unit water shut off valves to be located in unit closets, with valves exposed outside sheetrock or inside walls with access panels furnished and installed by others (meters/jumpers to be furnished by others).

Water hammer arresters installed per code.

ProSet sleeve system or equal to be used at post tension decks only.

EXCLUDED FROM BASIC PLUMBING BID

Trench drains and drain line shown on page C0300, outside the building foundation, on west side of building.

Storm detention vault, ladders, or manhole.

Excavation, backfill, fill materials, pea gravel, sand or washed rock inside or outside of building.

Placement of sand/oil or pump basins in slab on grade.  Export of trench spoils. Drain rock or filter fabric.

Pumps systems for storm drain piping.  Storm sewer inverts are assumed to be low enough for detention

vault to drain by gravity.

Foundation or footing drains outside buildings.   Drain mat or adaptors at foundation walls.

Sub-slab drainage piping.

Exterior downspout piping.

Emergency electrical shutoffs by others.

Pipe protection or bollards for pipes coming down walls or columns in parking garage.

Stud shoe or strap reinforcement of wall studs in plumbing walls.

Housekeeping pad for water heaters.

Exterior canopy, deck or roof downspouts, scuppers, gutters or overflows.

Thermal or acoustic insulation on waste, vent, drain or water piping (except as noted above for circulation main).

Wash machine drain pans.  Kitchen sink garbage disposals.

City water meters.    Unit water meters, jumper or unions.     Access panels.     Sewer hookup.     Gas meters.

EXCLUDED FROM BASIC PLUMBING BID (Continued)

Landscape or planter irrigation system, piping, controls or valves.

Waste disposal fees.

Hoisting of stalls, water heating equipment or roughin materials during roughin (load and off load by OWP).

Shower doors, shower curtains/rods. Sink hole cutouts. Garbage disposal supply.

OWP 2025			THE CORA		19 West Harrison Street

Exxel Pacific			    	 			Seattle, 98119

Labor codes:

100	Supervision

101	Takeoff and purchasing labor

110	Groundwork labor (includes waste, vent, water or foundation/storm drainage)

111	Waste, storm drain, condensate drain or water labor above slab on grade within the parking garage, amenity or retail spaces

112	can out labor

120	Waste, vent, storm drain or water roughin labor in the residential units

130	Finish or trim labor

140    Gas piping labor

141    water main and insulation labor

142	Mechanical room labor

Material codes:

210	Underground materials

211	Waste, storm drain, condensate drain or water labor above slab on grade within the parking garage, amenity or retail spaces

212	can out materials

220	Waste, vent, storm drain or water roughin material in the residential units

230	Finish or trim materials

240	Gas piping materials

241	water main and insulation material

242    Mechanical room roughin materials

Other codes:

600	  subcontractor

601	  engineering

602	  rental equipment

603	  permit and review fees

607	  Parking and other expenses